## Table 1. Components of the Disturbed Equilibrium Index (DEI)

| Block | Code | Indicator (normalized 0–1) | Description | Typical Data Source |
| --- | --- | --- | --- | --- |
| H (Human Pressure) | h<sub>LUC</sub> | Impervious surface / LULC change | % of land converted to urban/agriculture, surface sealing | Landsat, Sentinel-2, national LULC maps |
|  | h<sub>HA</sub> | Hydrological alteration | Dams, canals, blocked floodplains | River gauges, floodplain maps |
|  | h<sub>EXT</sub> | Extraction pressure | Groundwater/surface water withdrawal vs. recharge | Ministry of Water, GRACE data |
|  | h<sub>POL</sub> | Pollution load | Nutrient (N, P), COD/BOD, sediment | Environmental monitoring networks |
|  | h<sub>FRAG</sub> | Habitat fragmentation | Patch connectivity indices | GIS landscape metrics |
| A (Exogenous Stress) | a<sub>CA</sub> | Climate anomalies | SPI/SPEI, temperature/precipitation deviation | Meteorological services, ERA5 reanalysis |
|  | a<sub>EXT</sub> | Extreme events | Flood, drought, heatwave frequency/intensity | ETCCDI datasets, event catalogs |
|  | a<sub>GS</sub> | Geophysical sensitivity | Slope, erosion, subsidence potential | InSAR, DEMs, geological maps |
| R (Resilience) | r<sub>ER</sub> | Biodiversity/functional redundancy | Species richness, functional groups | IUCN, biodiversity surveys |
|  | r<sub>CN</sub> | Ecological/hydrological connectivity | Corridors, floodplain access | Connectivity indices, GIS |
|  | r<sub>SC</sub> | Storage/retention capacity | Wetlands, infiltration, aquifers | Soil/hydro data, wetland maps |
|  | r<sub>AC</sub> | Adaptive capacity & governance | Risk management plans, policy budgets, participation | Governance indices |
|  | r<sub>GE</sub> | Spatial/structural flexibility | Setback zones, NbS coverage | Planning documents, NbS assessments |

Caption: Components of the Disturbed Equilibrium Index (DEI) and candidate indicators normalized to a 0–1 scale. Each block represents human pressures (H), exogenous stressors (A), and resilience factors (R), with typical data sources suggested for operationalization in case studies.

## Table 2. DEI Classification Thresholds and Corresponding Management Actions

| DEI Range | Class | Ecosystem Status | Management / Policy Actions |
| --- | --- | --- | --- |
| DEI < 0.7 | Green (Safe) | Stable equilibrium; low pressures, adequate resilience | Routine monitoring; gradual investment in resilience (e.g., biodiversity protection, ecological connectivity). |
| 0.7 ≤ DEI < 1.0 | Yellow (Caution) | Early signs of imbalance; resilience still compensates | “Soft brakes”: reduce human pressures (optimize water use, land-use planning); strengthen buffers (wetlands, corridors). |
| 1.0 ≤ DEI < 1.3 | Orange (High Risk) | Approaching critical tipping points | “Rapid corrective action”: halt unsustainable withdrawals, emergency restoration, allocate risk management funds. |
| DEI ≥ 1.3 | Red (Crisis) | Systemic collapse or irreversible shift underway | “Rescue mode”: drastic interventions, redesign of governance, possible managed retreat or socio-economic transition. |

Caption: Example metadata of DEI indicators for Lake Urmia, Iran. Indicators cover human pressures, ecological reactions, and resilience factors, with definitions, units, normalization methods, and data sources indicated. This template can be replicated for other case studies.

## Table 3. Policy Matrix Linking DEI Classes to Governance Strategies

| DEI Class | Ecosystem State | Governance Priority | Policy / Management Tools |
| --- | --- | --- | --- |
| Green (Safe) DEI < 0.7 | Stable equilibrium; low stress | Long-term sustainability planning | Routine monitoring; biodiversity conservation programs; environmental education; low-cost nature-based solutions (NbS). |
| Yellow (Caution) 0.7 ≤ DEI < 1.0 | Early imbalance signals | Preventive adaptation | Water-use optimization; land-use zoning; reinforcement of ecological buffers; participatory governance mechanisms. |
| Orange (High Risk) 1.0 ≤ DEI < 1.3 | Approaching tipping point | Emergency risk reduction | Immediate restriction on resource withdrawals; moratorium on new dams/extractions; fast-track restoration projects; risk financing instruments. |
| Red (Crisis) DEI ≥ 1.3 | Collapse or irreversible shift | Systemic transformation | Crisis management task forces; redesign of governance frameworks; managed retreat/relocation; international cooperation; socio-economic safety nets. |

Caption: Policy matrix linking DEI threshold classes to governance strategies. The classification provides early-warning signals for decision-makers and associates each risk category with targeted management actions, from routine monitoring (Green) to systemic recovery programs (Red).

## Table 4. Comparative Summary of Case Studies under the Disturbed Equilibrium Index (DEI)

| Case Study | Historical Equilibrium | Main Disturbance Drivers | Key Consequences | DEI Interpretation |
| --- | --- | --- | --- | --- |
| Lake Urmia (Iran) | Stable hypersaline lake with rich biodiversity (brine shrimp, migratory birds) | Dam construction; irrigation withdrawals; recurrent drought | >80% shrinkage; salinity ~300 g/L; collapse of local biodiversity; salt storms affecting health | DEI ≈ 4.9 → Red (Chronic Collapse) |
| Aral Sea (Central Asia) | Balanced inflow–evaporation regime | Diversion of Amu & Syr Darya for irrigation; cotton monoculture | 90% shrinkage; fisheries collapse; desertification; dust storms | DEI ≈ 3.3 → Red (Persistent Collapse) |
| Shiraz Flood (Iran, 2019) | Seasonal floodplain corridors regulating flows | Urban encroachment; blocked drainage; Quran Gate road construction | 21 deaths; major urban damage; disruption of transport & services | DEI ≈ 11.1 → Red (Event Crisis) |
| Amazon Rainforest (Brazil) | Large-scale forest–climate equilibrium | Deforestation, fire; multi-year droughts; warming trends | Dieback risk; biodiversity loss; carbon sink weakening | DEI ≈ 3.7 → Red (Tipping Proximity) |
| Great Barrier Reef (Australia) | Stable coral–algae equilibrium | Heatwaves (bleaching); pollution; overfishing | Mass bleaching events; coral mortality; fishery & tourism decline | DEI ≈ 3.7 → Red (Crisis) |
| Hurricane Ian (USA, 2022) | Coastal resilience supported by wetlands & dunes | Coastal urbanization; extreme wind/surge | 150+ deaths; $100B+ damages; infrastructure collapse | DEI ≈ 3.4 → Red (Event Crisis) |

Caption: Comparative summary of six case studies analyzed under the DET/DEI framework. The table highlights historical equilibria, primary drivers of disturbance, consequences of disturbed equilibrium, and DEI interpretation, demonstrating the global applicability of the theory.